型号：365-A1

中山市老木子电器科技有限公司                                                          
水泵规格书

客　　户：九阳
[image: image1]Customer
[image: image2.png]35MM




品　　名：直流微型水泵 

Part name
型　　号：365-A1  

[image: image3.wmf]噪音测试仪

气泵下垫5cm厚的海绵垫�

检查仪器

气泵

Model
日　　期：2022.07.11
Date
页　　数：共 5页（包括封面）
Number of pages
	作　　成
	审　　核
	核　　准

	谭仲强
	 谭小玲
	牟伟


客户承认

Customer acknowledgment
	
	
	
	


	变更履历表

	NO.
	修订日期
	修订内容
	修订单位

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


	核准
	审核
	制定

	
	
	


1. 适用范围Application：

   本规格书适用于水泵365-A1之用.

   The specifications apply365-A1 use。

2. 测试条件Test condition
	2-1
	测试环境温度湿度

Environment test
	周围温度20℃，相对湿度65%RH为标准，如无争议发生可以在下述环境测试：周围温度5-45℃相对湿度30-80%RH。

The standard of environment test is the ambient temperature of 20℃ and relative humidity of 65%RH. Lf no disputes occur,the test can be conducted under the blow circumstance: the ambient temperature of 5~45℃ and relative humidity of 30~80%RH.

	2-2
	测試姿势（Position）
	水平（Horizontal）

	2-3
	额定负载（Load）
	空气压正常的气压环境下

	2-4
	使用温度范围 

Operation temperature range
	5 ~45ºC(Humidity to 5 to 45℃)

	2-5
	保存温度范围Preservation temperature range
	-20 ~60ºC(Humidity to -20 to 60℃)


3  性能 Function    
	3-1
	额定电压Rated voltage
	DC12.0V

	3-2
	使用电压范围

Operating voltage range:
	DC9.6V~14.4V

	3-3
	负载电流Rated current
	供电DC12V，水泵抽水时的最高电流≤420mA.
The highest current is less than 420mA while the pump for drawing water with DC12V.
	≤420mA 

Less than 
420mA 

	3-4
	空载流量 （水体）Water Flow
	在DC12.0V,过水流量大于1.5升/分钟。

 At DC12.0V, the water flow is more than 1.5L/min with no load. 
	≥1.5L/min


	3-5
	噪音Noise
	下垫5cm之海綿, 距噪音计30cm之处, 依DC 12V之电源,泵吸水时所测得之dB值小于50dB.

Putting a 5 cm sponge under the pump,and placing it about 30 cm away from the noise meter,The evaluated noise level shall be less then 50dB while pressurizing　with DC12.0V .
	≤50dB

Less than 
50dB

	3-6
	最大水压

Maximum pressure
	输入12.0V之电压时，水泵的最大水压大于1.5Kg.

The maximum pressure value shall be more than 1.5Kg while pressurizing with DC 12.0 V .
	≥1.5Kg
more than 1.5Kg

	3-7
	起动电压
启动电流

堵转电流
	输入10V之电压可使水泵启动.

启动电流小于1050mA

堵转电流：小于520 mA
	≤DC10V

≤1050 mA
≤520mA

	3-8
	扬程
	输入12.0V之电压时扬程1.2M
	≥1.2M


4  环境试验　Environmental Tests：

	4-1
	低温特性

Low-temperature characteristic
	放置在-20℃经72小时后, 再取出置于常温(20±5℃)中2小时无异状

Putting the pump in the temperature at negative -20℃ for 72 hours;then taking it out and putting it in a room temperature at 20±5℃ for 2 hours 

	4-2
	高温特性 

High-temperature characteristic
	在50℃经72小时放置后, 在放常温(20±5℃)中2小时测试无异状
Putting the pump in the temperature at 50℃ for 72 hours; then taking it out and putting it in a room temperature at 20±5℃ for 2 hours

	4-3
	高温高湿特性

 High-temperature & High humidity
	在温度50℃,湿度95%环境中经72小时取出置于常温(20±5℃)中2小时测试无异状Putting the pump in the temperature at 50℃ and the humidity at 95% for 72 hours;then taking it out and putting it in a room temperature at 20±5℃  for 2 hours 

	4-4
	寿命试验　

Life test
	水泵抽水工作，开45秒，停15秒为一个循环，30000次以后无异状.


5 特殊环境测试 Thermal test performance:

  在经过发下5-1和5-2项目之特殊环境测试后，要产品须可满足项目3-3至3-7之基本性能。

  After the pump has been tested through test 5-1 and 5-2, it must be suitable the basic 

  performance required by item 3-3 to 3-7.
	5-1
	低温环境测试 

Low-temperature environment test
	在温度0℃经五小时后,在此温度下测试无异状
Under a temperature of 0℃ for 5 hours.

	5-2
	高温环境测试 

High-temperature environment test
	在温度50℃经五小时后测试无异状.

Under a temperature of 50℃ for 5 hours


6 其它 Others:

	6-1
	端子强度 

The lead tension
	以0.5公斤法码与端子平行负载无异常者.

Hanging a 0.5 Kg static load on the terminal of the pump will not cause a damage on the pump.

	6-2
	落下实验 

Fall test
	使用标准的包装方式,在水泥地上将其从50cm的高度分别从正面,侧面,及边角自由落下,其性能及结构需无异常.

Basic performance and the struture shall not be defected after following test;With a standard packaging conditions,then dropping it at the front,side,and edges part from the height of 50 cm to the concrete floor.

	6-3
	外观 Appearance
	产品无不适当的外观和刮伤, 脏污及个塑料件之间的缝隙不超过0.5MM…等.

  

	6-4
	异常试验 Unusual test
	以DC12V连续8小时通电无燃烧,发烟现象.

There is no burning and smoking occurrence while the power is on with DC12V for 8 hours continuously.

	6-5
	马达温升
	气泵通电抽水时，马达表面达到的最高温度小于70度。

The pump for drawing water，motor surface temperature less than 70℃


7  噪音测试线路图Noise test circuit diagram:


8.成品外形图Finished Outline：

12.材料测试报告：详见附页



� EMBED  \* MERGEFORMAT ���
































	地址：广东省中山市东凤镇东凤大道南109号四楼
电话：13420411517



_1234567891.dwg
Administrator


