DISASSEMBLY

- Remove the screws located at the back panel.

- Remove the front panel screws—three at the top and three at the bottom.

- Carefully detach the touch panel, then lift the upper cover along with the styrofoam.

- Once the touch panel and upper cover are lifted, you will gain access to the fan motors, display board, PCB, anion generator, and swing motor.

- Open the PCB cover by unscrewing its mounting screws.

- Disconnect all wires from the PCB and gently lift the board. (pahanap nung video)

- Disconnect the wiring from the display board and lift it out. (pahanap nung video)

- Unscrew and remove the anion generator.

- Remove the screws securing the fan motor. (picture na lang na tinuturo yung screw sa fan motor)

- Carefully detach the grill cover from the cooling pad.(pahanap nung vid)

- Pull out the cooling pad by removing the lock stick. (pahanap nung vid)

- Unscrew and remove the water level sensor.

- Unscrew and remove the submersible water pump.

ASSEMBLY

- Install and secure the submersible water pump with screws.

- Install and secure the water level sensor.

- Mount and screw in the fan motor.

- Install the swing motor.

- Reinstall the display board and reconnect its wiring to the PCB.

- Reconnect all wiring to the PCB, following the correct connection layout.

- Carefully reinstall the PCB, ensuring all springs align with their corresponding touch panel buttons.

- Mount and screw in the ion generator.

- Reinstall the styrofoam.

- Reattach the upper cover, ensuring the swing motor aligns properly with the louvers.

- Reinstall the touch panel.

- Confirm that the upper cover is securely fitted on all sides.

- Reinstall the front panel screws (top and bottom).

- Reinstall the back panel screws.

- Reinsert the honeycomb cooling pad, making sure it is properly aligned and locked in place, then reinstall the grilles.