Assembly and Disassembly of Water Dispenser

Disassembly:

- Ensure that all stored water within the water dispenser is fully drained. Disconnect the Silicone hose connected to the water gallon and dispense water from the faucet until no water flow is observed.

(Video to Use: Removing Hose and Water)

- Drain the remaining water through the rear drain plug by opening it completely.

(Video to Use: Draining Water on Back)

- Remove the screws securing the rear cover plate and detach the plate to expose internal components.

(Video to Use: Removing Screws at the Back)

- Remove the screws located on the top cover and detach the top panel.

(Video to Use: Remove the Screws on the Top Part)

- Once the covers are removed, access the internal components and disconnect all wiring connections from the power switch, cooling switch, and heating switch.

(Video to Use: Removing the Water Tank Lid and Connections) 
(VIDEO EDIT: Properly Label each connector to ensure accurate re-installation)

- Detach the water tank lid and remove the water level sensor from the tank assembly.

(Video to Use: Removing the Water Tank Lid and Connections)

- Unscrew the PCB housing and disconnect all the wiring harnesses from the PCB.

(Video to Use: Remove the Screws on PCB Housing) 
	(Video to Use: Removing Cables on PCB)

- Removal of the internal partitions is required, locate the mounting screws at the bottom section. Remove the front panel and unscrew the metal brackets to allow removal of the water tanks and compressor assembly.

(Video to Use: Removing Metal Partition) 
(Video to Use: Removing Metal Partition 8.5)

- Remove the Water Pump by releasing it from the rubber locking mount and disconnect its wiring connections.

(Video to Use: Removing the Water Pump)

- After removing the screws securing the water tank, access the thermostat located underneath the water tank assembly.

(Video to Use: Removing the Thermostat)

- To Remove the hot water tank, disconnect the supply wiring and detach the thermodisc connected to the tank.

(Video to Use: Removing Hot Tank)

- Unscrew and Remove the Water faucets from the front panel to fully disengage the dispensing assembly.

(Video to Use: Removing Water Faucets)

ASSEMBLY:

- Secure the water tank lid properly. Ensure that the water level sensor and the corresponding hose are correctly inserted and positioned in their designated openings.

(Video to Use: Water Lid and PCB)

- Reconnect all wiring harness to the PCB based on the previously applied identification tags, ensuring correct terminal placement and secure connections.

(Video to Use: Water Lid and PCB)

- Install the Water Pump back into the rubber lock mount and verify that the pump is firmly seated. Reconnect the pump wiring.

(Video to Use: Taking Back Water Pump)

- Install the water faucets back onto the front panel using the appropriate screws.

(Video to Use: NOTE: Pa-reverse po ng Video 12: Removing of 
         Faucets)

- Reattach the fasten the condenser and top cover panel using the proper screws, ensuring all structural components are aligned and secured.

(Video to Use: Condenser and Top Part)